# PROM Installation Guide

# Lynx Time Code Module

	Optional	TIMELINE VISTA, INC.

1755 LA COSTA MEADOWS DRIVE, SUITE B

	Recommended	SAN MARCOS, CA 92069

TEL. 760-761-4440

q	Mandatory 	(FAX 760-761-4449)

SUPPORT@DIGAUDIO.COM

### DATE:	6/9/92

### MODEL:	Lynx Time Code Module

### REVISION:	All

### SERIAL NO:	All

### SOFTWARE:	PROM(s) U16 (512K) or U16 (256K) and U7 (256K)

REQUIRED TOOLS:

Static safe workstation	IC Extractor/Inserter or
	Phillips screwdriver	small screwdriver

Grounding wrist strap

REQUIRED PARTS:

Software Information Sheet
	Replacement PROM from TimeLine

PROCEDURE:

1.	Turn off the power to the Lynx Time Code Module. Disconnect all Cables.

- Place the Lynx on a static safe workstation. Ground yourself and the workstation antistatic mat.

- Remove the 6 phillips screws holding the cover to the chassis. Remove the top cover.

- Locate U16 and U7, the Lynx PROM(s). If the software supplied is a 512K PROM, only U16 is used. If you are using 256K PROMS, both U16 and U7 are used. See Figure 2.

Figure 1. Remove the Top Cover

Figure 2. PROM Location

PROM Installation Guide

PROCEDURE continued:

5.	Remove and replace the PROM(s). Be careful to correctly align Pin 1 and properly seat the PROM(s). NOTE: Rev A Printed Circuit Boards (Serial No. Up to 0769) have one Jumper, JP1 and can only be fitted with 256K PROMS. Rev B (Serial No. 0770-1453) and Rev C Printed Circuit Boards can be configured with either 256K or 512K PROM(s). Note the PROM size marked on the labels and then check Jumpers JP1, JP3, and JP4. Jumpers should be positioned as shown in Figure 3.

Warning

Use an IC inserter to remove and replace the PROM. If one is not available, use a small slotted screwdriver. Insert the screwdriver under the PROM, twist slightly and gently lift up.

Inserting the screwdriver incorrectly could damage surrounding components.

Figure 3. Jumper Configuration

6.	Replace the cover on the Lynx Time Code Module. Insert and tighten the 6 phillips screws. Reconnect the cables and turn on the power. The Lynx module will automatically detect the new software version. To verify that the upgraded software is operating, the new software version numbers will be displayed first after turning the power on.