# SERVICE BULLETIN

# No.	92-003

TIMELINE VISTA, INC.

1755 LA COSTA MEADOWS DRIVE, SUITE B

SAN MARCOS, CA  92069

Tel. 760-761-4440

(FAX 760-761-4449)

SUPPORT@DIGAUDIO.COM

RS 422  Interconnect Cable

### DATE:	2/06/92

### MODEL:	Lynx

### REVISION:	All

### SERIAL NO:	4285 – 4346

### SOFTWARE:	All

### DESCRIPTION:

These units have an RS422 cable in which  the shield drain wire is connected to the 9-pin D connector case at both ends of the cable.  This can cause RS422 communications problems.  The shield must be disconnected at one end.

REQUIRED TOOLS:

Static safe workstation	Phillips screwdriver
	Grounding wrist strap	Dike or cutter

PROCEDURE:

1.	On either connector, remove the two screws holding the connector shell in place.

2.	Locate the large black wire that connects the cable shield to the connector body.

Illustration 1. Ground Shield Termination

SERVICE BULLETIN
NO. 92-003

RS 422  Interconnect Cable, cont.

3.	Use the dikes or cutter to remove the shield drain wire between the metal connector case and 
	the edge of the jacket.

4.	Replace the connector shell.  Insert and tighten the two screws which hold the connector shell in place.